ESP32 FALL DETECTOR
Complete Pin Connection Sheet

One MPU6050 primary waist-sensor build | Follow this sheet for physical assembly

# 1. Complete signal and power connections

| No. | From pin | To pin | Wire/function | Instruction |
| --- | --- | --- | --- | --- |
| 1 | ESP32 3V3 | Breadboard + rail | Red power | 3.3 V supply rail |
| 2 | ESP32 GND | Breadboard − rail | Black ground | Common ground rail |
| 3 | MPU6050 VCC | Breadboard + rail | Red power | Sensor VCC; 3.3 V only |
| 4 | MPU6050 GND | Breadboard − rail | Black ground | Common ground |
| 5 | MPU6050 SDA | ESP32 GPIO 21 / D21 | Green I2C data | Do not connect to D22 |
| 6 | MPU6050 SCL | ESP32 GPIO 22 / D22 | Yellow I2C clock | I2C clock = 400 kHz |
| 7 | MPU6050 AD0 | Breadboard − rail / GND | Black address select | Address = 0x68 |
| 8 | MPU6050 INT | Not connected | — | Not used by current firmware |
| 9 | MPU6050 XDA | Not connected | — | Not used |
| 10 | MPU6050 XCL | Not connected | — | Not used |
| 11 | ESP32 GPIO 4 / D4 | Buzzer positive (+) | Green alarm signal | Use transistor if buzzer current is high |
| 12 | Buzzer negative (−) | Breadboard − rail / GND | Black ground | Common ground |
| 13 | ESP32 GPIO 2 / D2 | 220Ω resistor input | Purple LED signal | Resistor must be in series |
| 14 | 220Ω resistor output | LED anode (+) | LED current limiting | Longer LED leg is normally anode |
| 15 | LED cathode (−) | Breadboard − rail / GND | Black ground | Shorter leg/flat side normally cathode |

# 2. Breadboard rail connections

| Breadboard point | Connect to | Purpose |
| --- | --- | --- |
| Red/positive rail (+) | ESP32 3V3 | 3.3 V supply for MPU6050 |
| Red/positive rail (+) | MPU6050 VCC | Sensor power |
| Blue/negative rail (−) | ESP32 GND | Main ground |
| Blue/negative rail (−) | MPU6050 GND | Sensor ground |
| Blue/negative rail (−) | MPU6050 AD0 | Force I2C address 0x68 |
| Blue/negative rail (−) | Buzzer negative | Buzzer return path |
| Blue/negative rail (−) | LED cathode | LED return path |

# 3. USB and battery connections

| Component | Pin | Connect to | When used |
| --- | --- | --- | --- |
| ESP32 USB connector | USB cable | Computer USB port | Programming and first test |
| LiPo battery | Positive (+) | TP4056 B+ | Battery terminal only |
| LiPo battery | Negative (−) | TP4056 B− | Battery terminal only |
| TP4056 | OUT+ | ESP32 VIN/5V input | Battery-powered operation |
| TP4056 | OUT− | ESP32 GND | Battery ground |
| TP4056 | IN+/IN− or USB | USB 5 V charger | Charging input, according to module |

First test with USB power only. Add TP4056 and LiPo after the ESP32 + MPU6050 + buzzer + LED circuit works.

# 4. ESP32 pins intentionally unused in this build

Only the following ESP32 pins are used by the current firmware: 3V3, GND, D2, D4, D21 and D22. Do not connect other GPIO pins unless the firmware is changed.

# 5. Final physical checklist

☐ All GND points are common

☐ MPU6050 VCC is connected to 3V3, not 5 V

☐ SDA is connected to D21

☐ SCL is connected to D22

☐ AD0 is connected to GND

☐ Buzzer is connected to D4 and GND

☐ LED has a 220Ω series resistor

☐ USB test is completed before battery connection

☐ TP4056 B+ and B− are connected to battery polarity correctly

☐ TP4056 OUT+ and OUT− are connected to ESP32 VIN and GND

Firmware modes: Use REPLAY_MODE 0 for live physical MPU6050 input. Use REPLAY_MODE 1 only for the KFall replay demonstration.