ESP32 PRE-IMPACT FALL DETECTOR

Minimum Purchase List — Project Plan BOM | Dhaka estimated prices, August 2026

# Must-have components

| Component | Qty | Estimated total (BDT) | Purpose |
| --- | --- | --- | --- |
| ESP32 DevKit V1 (30/38 pin) | 1 | 450 | Main MCU |
| MPU6050 / GY-521 IMU | 2 | 400 | Waist sensor + spare / wrist ablation |
| TP4056 charger with protection | 1 | 100 | LiPo charging/protection |
| 3.7 V LiPo battery, 1000 mAh | 1 | 250 | Portable power and battery test |
| Active buzzer + red LED + resistors | 1 set | 60 | Alarm output |
| Perfboard + jumper wires + header pins | 1 set | 190 | Prototype/permanent wiring |
| TOTAL | — | 1,450 | Plan subtotal |

# Optional items from the plan

| Component | Estimated price (BDT) | Use |
| --- | --- | --- |
| Elastic belt / Velcro strap | 150 | Repeatable waist mounting |
| Small ABS enclosure | 200 | Wearable final build/photo |
| USB current meter | 350 | Direct power measurement |
| Spare ESP32 | 450 | Backup board |
| SD card module | 250 | Optional raw-signal logging |

# Required wiring

MPU6050 VCC → ESP32 3V3 | GND → GND | SDA → GPIO21/D21 | SCL → GPIO22/D22 | AD0 → GND (0x68) | Buzzer + → GPIO4/D4 | LED through resistor → GPIO2/D2 | All grounds common.

# Purchase order

প্রথমে must-have list কিনে breadboard prototype তৈরি করুন। দুইটি MPU6050-এর মধ্যে একটি spare হিসেবে রাখুন। Prototype সফল হওয়ার পরে optional belt, enclosure, current meter এবং spare board কিনুন।

Safety: TP4056 protection version ব্যবহার করুন। LiPo polarity উল্টো করবেন না। MPU6050-এ 5 V দেবেন না।